Test Report No.: RVT/ [[TCNo]]                          						Date: [[TCDate]]

ANNEXURE [[M03]]

Resistance to Vibration test as per Clause 7.5.2

Test Method:

Axes of orientation:

Subject the extinguisher to the vibration test in horizontal & vertical axes for the below parameters

Vibration parameters:

Frequency: 40 Hz, Amplitude: 0.5 mm Min.  Duration: 2 h (in each orientation)

Test Requirements:

Following Exposure to the vibration test,

- The extinguisher shall comply with the discharge requirements specified in 7.2

- Physical failure of components which require repair or replacement of the extinguisher and/or components before it can be returned to normal service shall be cause of rejection.

| [[ProductDesc]] | [[Cap1]] | [[Cap2]] | [[Cap3]] | [[Cap4]] | [[Cap5]] |
| --- | --- | --- | --- | --- | --- |
| Cylinder Sr. Nos. | [[SrNo1_4]] | [[SrNo2_4]] | [[SrNo3_4]] | [[SrNo4_4]] | [[SrNo5_4]] |

Observations:

| Cylinder Sr. Nos. | Min. discharge time |  | Min. Throw |  | % Discharge |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Requirement | Result | Requirement | Result | Requirement | Result |  |
| [[SrNo1_4]] | 8 sec. | [[R]:R10-15] sec. | 2 m. | [[R]:R3-5] m. | 85% | [[R]:P90-100] % | Pass |
| [[SrNo2_4]] | 8 sec. | [[R]:R10-15] sec. | 2 m. | [[R]:R3-5] m. | 85% | [[R]:P90-100] % | Pass |
| [[SrNo3_4]] | 13 sec. | [[R]:R22-25] sec. | 2 m. | [[R]:R3-5] m. | 85% | [[R]:P90-100] % | Pass |
| [[SrNo4_4]] | 15 sec. | [[R]:R25-28] sec. | 2 m. | [[R]:R5-6] m. | 85% | [[R]:P90-100] % | Pass |
| [[SrNo5_4]] | 15 sec. | [[R]:R28-30] sec. | 2 m. | [[R]:R6-7] m. | 85% | [[R]:P90-100] % | Pass |